Write Barrier Command Proposal for ATA-8 ACS

July. 31st, 2007

Revision 0.1

Author: Nathan Obr

Microsoft Corporation

One Microsoft Way

Redmond, WA. 98052-6399

USA

Phone : (425) 705-9157

E-Mail : natobr@microsoft.com

Table of Contents

| Revision History |  |  |
| --- | --- | --- |
| Date | Revision | Description |
| 2007-07-31 | 0.1 | Initial Draft |

# Introduction

This proposal discusses the details necessary for defining & implementing a Write Barrier command on an ATA device. The advantage to a Write Barrier command is having a method to communicate to a device the order that writes shall be destaged from a device’s internal cache to the device’s permanent media. This new feature, when used with a host that depends on ordered writes for data consistency; will guarantee data integrity and prevent data corruption in the case of power loss during drive operation while not adversely affecting performance.

This document provides a common understanding of the new concepts that Write Barrier introduces, provides a common language to discuss Write Barrier functionality, and proposes a new command to take advantage of such a device.

# Description of Consistency Problem on Storage

An application layer in a host that depends on the writes it creates to be committed to the non-volatile media of a device in a specific order cannot rely on the order that the writes were received by the device to convey write order information.  Currently there are no requirements or methods in the ATA specifications for devices to provide an environment that destages writes from internal cache in the order they arrived to the device as the order that the writes will be committed to permanent media.  The destaging issue seen in the devices internal cache can be seen anywhere in the system that queuing or caching occurs.

Multithreaded hosts that have multiple I/O generators that are scheduled in time slices and supported by storage subsystems that are also multithreaded and queued and provide no guarantee in the order of that I/O operations occur.  With the addition of NCQ, SATA provides another opportunity of I/O reordering within the device before writes make it to a device’s non-volatile media.  In an environment of no delivery order guarantee applications are responsible for ensuring order of I/O delivery by waiting for completion status on any command that must be serialized before sending down other commands in the same series.

However, a device with a buffer or volatile write cache may commit to permanent media writes that the device has received in an order that is optimized for performance rather than the order that the writes were received. Consequently even though an application has ensured the device has received the I/O in order, a device with a write cache may commit the I/O to the media in a random order thwarting the intent of the application to serialize the commitment of I/Os in order to the media.

The current approach to preventing this from happening is to make sure that no two serialized commands are ever in the devices write cache at the same time by ensuring that all previous writes have completed before the current write through performing a FLUSH after the targeted write has completed.  Unfortunately this causes all data to be committed to the device’s permanent media.  However, this operation is slow and hurts performance.  Consequently FLUSH is largely avoided by many implementations leaving the window of opportunity for data inconsistency open.

Alternately an application can serialize all of its ordered writes throughout the entire host and mark each one as a Forced Unit Access (FUA) write ensuring that each write is committed to permanent media when it is received by the device.  Although this doesn’t cause all writes buffered in a device to be committed to disk, the serialization from the application layer all the way down to the device’s permanent media is also very slow and FUA is also largely avoided by many implementations.

In order to avoid the performance penalties incurred by FLUSH and FUA, a solution is needed that allows a device to commit writes to non-volatile media with minimal impacts on performance.  This proposal suggests the solution is a Write Barrier command implementation.

# Description of Write Barrier

A Write Barrier is a command that creates a method for the host to provide ordered write information to the device by creating barriers across which queues and caches in between the application and the non volatile media may not reorder writes.  Write Barriers allow the application to overcome the performance reordering of write cache destaging algorithms.  By placing Write Barriers between only the write I/O that need to be ordered, the device continues to have the freedom to perform performance optimizations on the order it commits the writes bounded by any two Write Barriers. It is intended that Write Barrier does not specifically impact the existing optimizations within a device to re-order read I/O.

The intent of this proposal is to offer a definition for Write Barrier that ensures consistency of ordered writes while protecting the device’s ability to make performance optimizations as a preferable alternative to the current behavior of relying on the durability of all order sensitive application data on the drive through FLUSH and FUA commands.

# Write Barrier Feature Proposal

The primary focus of the proposed Write Barrier command is to enable the host to share write order information with the device, which is a new concept to the ATA standard. Consequently many of the concepts that will be used in discussion and in the remainder of this document are also new.  This section establishes the goals and requirements for the Write Barrier proposal, defines terminology for new Write Barrier concepts, and then illustrates the terminology using scenarios that typify Write Barrier usage.

## Proposed Solution Requirements

    - Write Barrier Feature Requirements

The purpose of Write Barrier Command is to create a mechanism by which the host may share write order information with the device.   Requirements for the Write Barrier proposal break down into the mechanism’s ability to deliver the write order information, the write order information’s ability to convey what the host is requesting, and the expected devices behavior when it receives the write order information.

The Write Barrier mechanism must:

- Be discoverable

- Provide a mechanism to distinguish between Write Barrier sensitive writes and Write Barrier insensitive writes.

- Provide a mechanism to require that the Write Barrier set is permanent on the media when the Write Barrier is completed

- Continue to allow write re-ordering by the device between Write Barriers

- Provide a mechanism that identifies Write Barrier Sets to be grouped that allows for multiple Write Barrier generators to specify  Write Barriers independent of each other

The write order information must:

- Be completely represented by the order in which the Write Barrier commands are delivered to the device.  This means the Write Barrier commands themselves are never delivered to the device out of order

- Not be affected by FUA or FLUSH commands

The device must:

- Be able to implement Write Barrier without a significant negative performance impact when compared to similar models of ATA drive that does not implement Write Barrier.

- Be able to implement Write Barrier without a significant increase in the complexity of the firmware of the device

Assumptions:

- It is the host’s responsibility to manage multiple IO access to the same sectors.  No device is required to honor multiple reads and writes to the same sector in the order that an arbitrary application issued the reads and writes.  The device is only responsible for honoring the reads and writes in the order they were received by the device.

## Proposed Definition of Terms for Solution

Write Barrier Set The write barrier sensitive commands, tagged with the same Write Barrier Group, bound by two Write Barrier commands tagged with the same Write Barrier Group.

Write Barrier Group A tag that is used on write barrier sensitive writes and Write Barrier commands to separate independent Write Barrier Sets that exist at the same time.

## Solution Overview: Scenarios and Illustrations

In the examples below an IO pattern is established and then transformations will be performed on the IO pattern to illustrate legal methods for destaging the IO from a queue or cache.

Each scenario carries over the changes from the illustration before.  There is no operational dependency between any of the transformations.

    - Scenarios

4.3.1.1 Sample IO Pattern

This illustration establishes a set of data to perform reorder operations on in the following examples.

4.3.1.2 Write Barriers limit write movement

Write IO may not be reordered across a Write Barrier from either direction.  The groups of Write IO contained within Write Barrier Commands are called Write Barrier Sets.

4.3.1.3 Write Barriers don’t restrict read movement with a Write Barrier Set

Any IO can be reordered within a Write Barrier

4.3.1.4 Write Barriers limit only Write Barrier Sensitive Writes to a Write Barrier Set

Reads and Write Barrier Insensitive Writes can be reordered independent of all Write Barriers

4.3.1.5 Write Barriers don’t restrict read or Write Barrier Insensitive Writes movement anywhere

Reads and Write Barrier Insensitive Writes can be reordered independent of all Write Barriers and be part of any Write Barrier Set

4.3.1.6 Writes can be reordered within a Write Barrier Set

Writes can be performed in any order as long as they do not cross a Write Barrier and become a part of a different Write Barrier Set.

4.3.1.7 Distinguishing Write Barrier Sets from multiple IO sources provides better destaging options, but not distinguishing Write Barrier Sets from multiple IO sources is still Write Barrier compliant.

Recognizing separate Write Barrier commands and tags from separate I/O sources allow for the device to perform destaging operations on larger Write Barrier sets (in this case recognizing that I/O 11 can be moved behind I/O 25 or that I/O 14 can be moved after I/O 12).  However not distinguishing separate Write Barrier commands and tags from separate IO sources just create smaller Write Barrier Sets (in this case {11,12}, {22,23,14,24}, {25}, {15,17,27}) which limit the transformations which can be performed.  No transformation that can be performed on the smaller Write Barrier Sets violates any rules of the larger Write Barrier Sets.

# Proposed Changes to ATA-8 ACS

The following ATA-8 ACS sections require modification:

- The Identify Device command is modified to be able to identify device support for the new command set and power mode.

- The Device Configuration Overlay feature set is used to control device behavior for the Write Barrier command.

## Identify Device Data

Determining whether a given ATA HDD supports Write Barriers is a straight forward process. One reserved bit of one of the ATA-8 ACS IDENTIFY DEVICE words is used by the HDD during device enumeration to indicate support.  The chosen IDENTIFY DEVICE word would be augmented with the following description:

Bit [TBD1] 	when set to one indicates that the device supports the Write Barrier command and performs Write Barrier destaging from the devices internal caches based on this information.

Bit [TBD2]	when set to one indicates that the device destages from its write cache in the same order that the device received the command.

## Device Configuration Overlay

Likewise, one bit of one of the ATA-8 ACS reserved DCO words is used to indicate to the device that the device may indicate Write Barrier support.  The chosen DCO word would be augmented with the following description:

Bit [TBD3]	indicates whether the device may support of the Write Barrier. When set to one, the device is allowed to report support for the Write Barrier command. When cleared to zero, the device shall not indicate support or support the Write Barrier.

## WRITE DMA

At least one Write Command must be able to be tagged with a Write Barrier Group to indicate Write Barrier sensitivity.  The logical place would be to add the Write Barrier Group to the Feature Field of the Write DMA command.  The following addition would be made:

Features

Bit 7:4 Write Barrier Group – Provides a group tag number to separate different Write Barrier sensitive groups from each other.  The value 0 indicates this write is not Write Barrier sensitive.  All other values indicate a unique Write Barrier Group and that the write is Write Barrier Sensitive to only other writes tagged with that Write Barrier Group.

# Proposed New Commands for ATA-8 ACS

The WRITE BARRIER command is introduced to contain the Write Barrier fields necessary for conveying Write Barrier information.

## WRITE BARRIER Command

Write Barrier Group – Provides a group tag number to separate different Write Barrier sensitive groups from each other.  The value 0 is reserved.  All other values indicate that the Write Barrier affects only writes tagged with that Write Barrier Group.

FUA – this indicates that the WRITE BARRIER command can not be completed until the Write Barrier Set is indicated by the Write Barrier Group is permanently on the media.

#### Normal Outputs

See Table 64.

#### Error Outputs

See Table 78